CPU PROJECT

Three-Week Development Journal

Transistor-Level Logic, LTspice Simulation, PyRTL Conversion,
and Physical Adder Design

| Student | Goodwin Chen |
| --- | --- |
| Project period | Three weeks, Summer 2026 |
| Estimated work completed | Approximately 42 hours |

A record of completed design, simulation, debugging, and documentation work

Project Description

I am building an 8-bit CPU from the transistor and gate level upward. The current physical hardware milestone is an 8-bit ripple-carry adder constructed from discrete NPN transistors. To reach that point, I designed and tested the required logic gates, full adders, multiplexers, shifting circuits, and an eight-function ALU in LTspice. I also developed a PyRTL-to-LTspice converter that translates digital designs into LTspice netlists and expanded it to support arithmetic, multiplexers, registers, clocked circuits, and feedback. For the physical implementation, I created a detailed one-bit full-adder layout in Fritzing, planned the complete eight-stage adder assembly, and prepared a professional bill of materials and budget. The adder serves as the arithmetic foundation for the larger CPU documented in the GitHub repository.

Journal scope: This journal records work already completed. Because I did not track every session with a timer, the hour totals are professional estimates based on the amount of design, simulation, debugging, and documentation completed.

Progress at a Glance

| Week | Primary Focus | Major Deliverables | Hours |
| --- | --- | --- | --- |
| 1 | LTspice logic and ALU design | Transistor-level gates, full adder, ripple-carry structure, shifts, multiplexers, and complete 8-bit ALU | 16 |
| 2 | PyRTL-to-LTspice converter | Operation mapping, reusable cells, register support, DFF validation, 8-bit register, and 4-bit counter | 15 |
| 3 | Physical layout and build planning | Fritzing full-adder layout, 8-bit assembly plan, truth-table verification, BOM, and budget | 11 |
| Total |  | Estimated completed work | 42 |

| WEEK 1 ¦ Transistor-Level Logic and Full ALU Assembly | 16 HOURS |
| --- | --- |

OBJECTIVE

Design and simulate the transistor-level combinational logic required for an 8-bit arithmetic logic unit, then combine the individual functions through multiplexers into one complete ALU schematic.

Work Completed

Created LTspice schematics for the inverter, AND, OR, XOR, and other fundamental logic functions needed by the datapath.

Built and tested a one-bit full adder with A, B, and carry-in inputs and separate SUM and carry-out outputs.

Expanded the one-bit design into a multi-bit ripple-carry structure by routing each stage’s carry output into the next stage’s carry input.

Implemented subtraction through two’s-complement logic, including inversion of the B input and the required carry-in behavior.

Designed separate shift-left and shift-right paths, including the grounded bit introduced at the empty end of each shift.

Built multiplexer trees to select among arithmetic, logic, and shift operations.

Assembled the complete eight-function, 8-bit ALU schematic and organized the buses for A, B, operation-select inputs, results, and carry signals.

Debugged wiring faults involving the least significant bit, ground connections, shared branches, and overdefined parallel voltage sources.

Verified representative operations in LTspice, including 5 + 3, 5 - 3, XOR, and shifting.

Design Documentation Produced

Individual transistor-level gate diagrams

One-bit full-adder schematic

Eight-bit ripple-carry adder structure

Shift-left and shift-right diagrams

Multiplexer selection network

Complete 8-bit ALU assembly

LTspice waveform captures showing correct outputs

Technical Reflection

This stage established the logic foundation of the CPU. The most important lesson was that correct Boolean equations are not enough by themselves; bus ordering, carry direction, grounding, and node connectivity must also be exact. Debugging the ALU required following individual bits through the circuit and checking whether each wire represented the intended signal.

| WEEK 2 ¦ PyRTL-to-LTspice Converter and Sequential Logic | 15 HOURS |
| --- | --- |

OBJECTIVE

Automate the translation of PyRTL hardware descriptions into LTspice circuits and expand the converter beyond isolated combinational gates to support reusable arithmetic cells, multiplexers, registers, clocks, and feedback.

Work Completed

Developed a Python converter that reads a PyRTL working block and writes an LTspice .cir netlist.

Started with support for AND, OR, XOR, NAND, NOT, and direct wire operations using an embedded transistor-level gate library.

Added optional PyRTL synthesis so complex combinational expressions could be lowered into simpler logic operations.

Expanded direct operation mapping for multiplexers, constants, bus wiring, concatenation, bit selection, arithmetic, and comparisons.

Created reusable LTspice standard cells for INV, NAND2, AND2, NOR2, OR2, XOR2, XNOR2, MUX2, FULLADDER, and DFF.

Added register handling so a multi-bit PyRTL register is automatically expanded into one D flip-flop per bit with a shared clock.

Generated and tested example circuits including a full adder, synthesized 2-bit adder, 8-bit register, register with load behavior, and 4-bit synchronous counter.

Diagnosed a race-through problem in the original transistor-level DFF when the output fed directly back through an adder into its input.

Replaced the large-circuit register cell with LTspice’s native edge-triggered DFLOP while keeping the combinational gates transistor-level.

Verified the corrected counter waveform as it advanced through the expected binary sequence.

Converter Capabilities Demonstrated

| Category | Supported Behavior |
| --- | --- |
| Basic logic | AND, OR, XOR, NAND, NOT |
| Wiring and buses | Direct wires, constants, bit selection, and concatenation |
| Combinational modules | Multiplexers, adders, subtractors, multipliers, and comparators |
| Sequential modules | Registers, shared clock generation, initialization, and feedback |
| Validation circuits | 8-bit register and 4-bit synchronous counter |

Technical Reflection

The counter became an important regression test because it combined arithmetic, clocked state, and immediate feedback. A DFF that appeared correct in isolation still failed when its output changed the next input during the same clock transition. Fixing the reusable DFF cell corrected the underlying sequential behavior without adding counter-specific logic, preserving the converter’s generalized architecture.

| WEEK 3 ¦ Fritzing Layout, Physical Full Adder, and Build Planning | 11 HOURS |
| --- | --- |

OBJECTIVE

Translate the simulated logic into a physical breadboard plan using discrete NPN transistors, verify the one-bit full-adder structure, and prepare the complete parts and assembly plan for an 8-bit ripple-carry adder.

Work Completed

Studied the collector, base, and emitter roles of the 2N3904 NPN transistor and how NPN stages behave as switches and inverters in resistor-transistor logic.

Used Fritzing to plan transistor placement, breadboard routing, power rails, resistor placement, switches, and LED indicators.

Developed and refined a complete one-bit full-adder layout with clearly identified A, B, carry-in, SUM, and carry-out signals.

Checked the full-adder behavior against all eight input combinations in the truth table.

Planned the full eight-stage ripple-carry assembly by repeating the one-bit module and connecting each carry-out to the next stage’s carry-in.

Selected breadboards, 22 AWG solid-core wire, 2N3904 transistors, resistor values, LEDs, three-pin SPDT switches, and a 4.8 V rechargeable power source.

Prepared a GitHub-ready bill of materials with product links, quantities, purposes, and estimated prices.

Calculated an estimated component total of $83.93 before tax and shipping and recommended a working budget of approximately $90.

Full-Adder Verification Table

| A | B | Cin | SUM | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Eight-Bit Assembly Structure

FA0: A0 + B0 + Cin -> S0 + C1 FA1: A1 + B1 + C1 -> S1 + C2 FA2: A2 + B2 + C2 -> S2 + C3 ... FA7: A7 + B7 + C7 -> S7 + Cout

Technical Reflection

Moving from simulation to Fritzing required a different type of precision. A circuit can be logically correct but still fail physically if transistor pins are reversed, breadboard rows are misunderstood, power rails are disconnected, or wires are difficult to trace. The Fritzing plan and parts list converted the electrical design into a buildable physical project while keeping the larger CPU goal intact.

Current Project Status

LTspice logic library: Completed and used to construct transistor-level gates, arithmetic, shifting, and selection logic.

8-bit ALU schematic: Completed and tested with representative arithmetic and logic operations.

PyRTL-to-LTspice converter: Generalized across major combinational operations and register-based sequential circuits; memory operations remain outside the current implementation.

Sequential validation: A working DFF path, 8-bit register generation, and a correct 4-bit synchronous counter waveform have been demonstrated.

Physical full-adder design: A detailed Fritzing plan and full truth-table verification are complete.

8-bit adder planning: The carry-chain structure, breadboard layout approach, component list, and budget are documented.

Project budget: Estimated component total: $83.93 before tax and shipping; recommended budget: approximately $90.

Summary of Completed Deliverables

Transistor-level diagrams for the major logic functions

One-bit full-adder and multi-bit ripple-carry diagrams

Multiplexer networks and complete 8-bit ALU assembly

LTspice waveforms for combinational and sequential tests

Generalized PyRTL-to-LTspice conversion software

Register, DFF, and synchronous counter test circuits

Fritzing physical-layout plan for the full adder

Eight-stage adder assembly plan

Professional parts list with prices and links

Overall result: The project has progressed from individual transistor-level gates to a complete simulated ALU, a reusable conversion tool, and a physically planned full-adder module that can be repeated into an 8-bit ripple-carry adder while remaining part of the larger CPU repository.